# Creating a Smart Layout for nesting die files

You can use the Smart Layout feature to automatically gang die files for production.

- Select File > Import > Die.

- In the Import Die dialog box, browse to the location of the die file, select the file, and then click Open (Macintosh) or Import (Windows).

- When the die appears in the Die folder on the Content tab of the Information window, drag it to the layout view. If you have already imported a substrate to the layout view, the software will center the die on the substrate.

- From ShopMap, add a substrate.

- In the job properties pane, select SmartLayout for dies.

- Select an item under Die and, in the Parameters section, provide values for Minimum Amount and Artwork Rotation.

- Select an item under Sheets, and provide values for Fill Direction, Start Corner, and Margins.  
Grain direction is dictated by the specified substrate and cannot be modified.

- If desired, select one or more of the Options:

  - Guillotine Cut (align items for the minimum number of cuts in the vertical direction)

  - Clearance (enter a value)

- To view the results of your settings, click Create Solution.

- If desired, change the settings to produce different ganged layouts and click Create Solution to view the results.

- Optional: To capture a solution without applying the results, click Capture Selection.

- Optional: To generate a job report, click Save Report and specify where to save the .txt file.

- Under Solutions, select the solution that produced the best layout for your purposes.

- Click Apply.